DATA COMMUNICATION & COMPUTER NETWORKS

- A network of networks is called an internetwork, or simply the internet.

- A computer network is a collection of computing devices that are connected with each other for the purpose of information and resource sharing among a wide variety of users.

- The multiple terminals connected to the network sharing the network resources are known as nodes.

- A Personal Area Network (PAN) is smallest network which is very personal to a user. This may include Bluetooth enabled devices or infra-red enabled devices. PAN has connectivity range up to 10 meters.

- A computer network spanned inside a building and operated under single administrative system is generally termed as Local Area Network (LAN).

- The Metropolitan Area Network (MAN) generally expands throughout a city such as cable TV network.

- the Wide Area Network (WAN) covers a wide area which may span across provinces and even a whole country.

- Ethernet shares media. Network which uses shared media has high probability of data collision. Ethernet uses Carrier Sense Multi Access/Collision Detection (CSMA/CD) technology to detect collisions.

- Traditional Ethernet uses 10BASE-T specifications.

- The number 10 depicts 10MBPS speed, BASE stands for baseband, and T stands for Thick Ethernet.

- Fast Ethernet uses CSMA/CD technique for wired media sharing among the Ethernet hosts and CSMA/CA (CA stands for Collision Avoidance) technique for wireless Ethernet LAN.

- Open System Interconnect is an open standard for all communication systems. OSI model is established by International Standard Organization (ISO).

- Cryptography is a technique to encrypt the plain-text data which makes it difficult to understand and interpret.

- Secret Key Encryption: Data Encryption Standard (DES).

- Public Key Encryption: Rivest-Shamir-Adleman (RSA).

- Message Digest: MD5

- Computer Network Hardware Components:

  - Servers are high-configuration computers that manage the resources of the network.

  - Clients are computers that request and receive service from the servers to access and use the network resources.

  - Peers are computers that provide as well as receive services from other peers in a workgroup network.

  - Transmission media are the channels through which data is transferred from one device to another in a network.

  - Connecting devices act as middleware between networks or computers, by binding the network media together. Some of the common connecting devices are: Routers, Bridges, Hubs, Repeaters, Gateways and Switches.

- Connecting devices provides an intermediaries for communication between different networks or computers by linking various devices, this helps to travel data across multiple systems and network efficiently.

- Hubs: It's a basic device, which connects multiple devices in a network. It operates on the physical layer of the OSI model and broadcasts data to all connected devices.

- Switches: It's a more advanced version of the hub, which intelligently directs data packets to the specific device improving network efficiency. It reduces network collisions compared to hubs.

- Router: It's a device, that routes data packets between different networks like between LAN and the internet, this operates on the network layer of the OSI model and can also assign IP addresses, due to which it is capable of performing extra functions like security filtering and quality of service (QoS).

- Gateway: This device connects two different networks, of different protocols or architectures.

- Repeaters: This device is used to amplify or regenerate signals.

- The Receiver is the component that receives and decodes the signals transmitted over the medium.

- A device that eliminates a section of cabling or implements a state of access for network devices, including PCs, hubs, and switches.

- Connectors can be famous for their physical presentation and mating features, including jacks and attachment (male connectors) or attachments and ports (female connectors).

- Connectors are used to connect the guided (wired) transmission media to devices like the hub, server, workstations etc.

- Types of Connectors:

  - Twisted Pair Cable Connectors:

- UTP (Unshielded Twisted Pair):  RJ45 (RJ represents Registered Jack)

- STP (Shielded Twisted Pair)

  - Coaxial Cable Connector: Bayonne Neill Concelman (BNC)

  - Fiber-optic Cable Connector:

- The Subscriber Channel (SC) connector is used for cable T.V. It uses a push/pull locking system.

- The straight-tip (ST) connector is used for linking wire to networking devices. It uses a bayonet locking system and is more reliable than SC.

- MT-RJ is a connector that is a similar size to RJ45.

- What is the function of the DB-25 connector?

Ans. Serial communication

- Switches are networking devices operating at layer 2 or a data link layer of the OSI model. They connect devices in a network and use packet switching to send, receive or forward data packets or data frames over the network.

- It uses MAC addresses (addresses of medium access control sublayer) to send data packets to selected destination ports.

- Types of Switches:

  - Unmanaged Switch

  - Managed Switch

  - LAN Switch

  - Power over Ethernet (PoE) switches

- Repeaters are network devices operating at physical layer of the OSI model that amplify or regenerate an incoming signal before retransmitting it.

- According to the types of signals that they regenerate, repeaters can be classified into two categories:

  - Analog

  - Digital

- According to the types of networks that they connect, repeaters can be categorized into two types:

  - Wired

  - Wireless

- According to the domain of LANs they connect, repeaters can be divided into two categories:

  - Local

  - Remote

- A gateway is a network node that forms a passage between two networks operating with different transmission protocols.

- On basis of direction of data flow, gateways are broadly divided into two categories:

  - Unidirectional

  - Bidirectional

- On basis of functionalities, there can be a variety of gateways, the prominent among them are as follows:

  - Network

  - Cloud Storage

  - Internet to orbit

  - IOT

  - VoIP trunk

- Bridges are used to connect two subnetworks that use interchangeable protocols.

- Types of Bridges:

  - Transparent: It is also called learning bridges. Bridge construct its table of terminal addresses on its own as it implements connecting two LANs. It facilitates the source location to create its table. It is self-updating. It is a plug and plays bridge.

  - Source routing: This sending terminal means the bridges that the frames should stay. This type of bridge is used to prevent looping problem.

- A network socket is a software component within a computer network node that acts as an endpoint for delivering and receiving data

- Types of Sockets:

  - Datagram: Connection-less

  - Stream: Connection-oriented

- Network Socket Client Server Interaction:

  - create()	To construct a socket

  - bind()	              It's a socket identifier, similar to a phone number for   contacting someone.

  - connect()	Are you ready to make a connection?

  - listen() 	Prepared to send a message

  - accept()	Confirmation is similar to accepting a call from a sender.

  - write()		To send data

  - read()		To receive data

  - close()		To make a relationship permanent

- A network interface card (NIC) is a hardware component without which a computer cannot be connected over a network.

- NIC is both a physical layer and a data link layer device

- NIC cards are of two types:

  - Internal:

- Peripheral Component interconnect

ii.    Industry Structure Architecture

  - External:

- Wireless

- USB based

- Which layer of the OSI model does a NIC operate at?

Ans. Data Link

- These are tiny electrical components that are inserted into computers or other devices to provide network connectivity. To send digital signals from the computer across the network, NIC might serve as a translator between the computer and the latter.

- Advantages:

  - Faster Network speed

  - Reliability

  - Security

  - Compatibility

  - Low CPU usage

- Disadvantages:

  - Limited transfer speed

  - Limited Compatibility

  - Driver issues

  - Cost

  - Power Consumption

  - Maintenance

  - Security risks

- A port is a physical docking point using which an external device can be connected to the computer.

- A Network Topology is the arrangement with which computer systems or network devices are connected to each other.

- Point-to-point Protocol (PPP) is a communication protocol of the data link layer that is used to transmit multiprotocol data between two directly connected (point-to-point) computers.

- Bus Topology.

- What type of cable is typically used in a bus topology?

Ans. Coaxial

- In a bus topology, how is data transmitted?

Ans. In one direction only

- Star Topology: A star topology is constructed with each node such as offices, printers, laptops, servers, etc., connected directly to a central computer known as a network switch.

- What type of cabling is commonly used in star network topologies?

Ans. Coaxial, Twisted Pair, Fiber Optic

- Ring Topology.

- Mesh Topology.

- A tree network topology hierarchically links computers and requires data to circulate through the branches.

- Which layer of the OSI model does a hybrid network topology primarily operate at?

Ans. Physical Layer

- OSI Model:

  - Physical Layer − Its function is to transmit individual bits from one node to another over a physical medium.

  - Data Link Layer − It is responsible for the reliable transfer of data frames from one node to another connected by the physical layer.

  - Network Layer − It manages the delivery of individual data packets from source to destination through appropriate addressing and routing.

  - Transport Layer − It is responsible for delivery of the entire message from the source host to destination host.

  - Session Layer − It establishes sessions between users and offers services like dialog control and synchronization.

  - Presentation Layer − It monitors syntax and semantics of transmitted information through translation, compression, and encryption.

  - Application Layer − It provides high-level APIs (application program interface) to the users.

- Which device operates at the physical layer of the OSI model?

Ans. Hub

- Physical layer in the OSI model plays the role of interacting with actual hardware and signaling mechanism.

- The media over which the information between two computer systems is sent, called transmission media.

  - Guided Media: Wired

  - Unguided Media: Wireless

- The speed of transmission of information is said to be the channel capacity.

- Multiplexing is a technique to mix and send multiple data streams over a single medium.

- Multiplexer (MUX) for multiplexing the streams and sending them on a medium, and de-multiplexer (DMUX) which takes information from the medium and distributes to different destinations.

- Switching is a mechanism by which data/information sent from source towards destination which are not directly connected.

- When digital data is converted into a bandpass analog signal, it is called digital-to-analog conversion. When low-pass analog signal is converted into bandpass analog signal, it is called analog-to-analog conversion.

- Bandpass: The filters are used to filter and pass frequencies of interest. A bandpass is a band of frequencies which can pass the filter.

- Low-pass: Low-pass is a filter that passes low frequencies signals.

- Which of the following is NOT an example of analog transmission?

Ans. Fiber optic cables

- What is the primary characteristic of analog transmission?

Ans. Continuous

- The transmission media is nothing but the physical media over which communication takes place in computer networks.

  - Magnetic Media

  - Twisted Pair Cables  Shielded & Unshielded

  - Coaxial Wires

  - Power Lines

  - Fiber Optics

- STP cables comes with twisted wire pair covered in metal foil. This makes it more indifferent to noise and crosstalk.

- UTP has seven categories, each suitable for specific use. In computer networks, Cat-5, Cat-5e, and Cat-6 cables are mostly used. UTP cables are connected by RJ45 connectors.

- Wireless transmission is a form of unguided media.

- Wireless communication involves no physical link established between two or more devices, communicating wirelessly. Wireless signals are spread over in the air and are received and interpreted by appropriate antennas.

- What frequency range do microwaves typically operate in?

Ans. 1GHz – 10GHz

- What is attenuation in the context of data transmission?

Ans. Loss of signal strength

- Which type of impairment is caused by external electromagnetic interference?

Ans. Noise

- What is distortion in data transmission?

Ans. Change in the shape of the signal

- Multiplexing is a technique by which different analog and digital streams of transmission can be simultaneously processed over a shared link. Multiplexing divides the high capacity medium into low capacity logical medium which is then shared by different streams.

- When multiple senders try to send over a single medium, a device called Multiplexer divides the physical channel and allocates one to each. On the other end of communication, a De-multiplexer receives data from a single medium, identifies each, and sends to different receivers.

  - Frequency division M: When the carrier is frequency, FDM is used. FDM is an analog technology. FDM divides the spectrum or carrier bandwidth in logical channels and allocates one user to each channel.

  - TDM: TDM is applied primarily on digital signals but can be applied on analog signals as well. In TDM the shared channel is divided among its user by means of time slot.

  - WDM

  - CDM: Multiple data signals can be transmitted over a single frequency by using Code Division Multiplexing. FDM divides the frequency in smaller channels but CDM allows its users to full bandwidth and transmit signals all the time using a unique code. CDM uses orthogonal codes to spread signals.

- Switching is process to forward packets coming in from one port to a port leading towards the destination.

- When two nodes communicate with each other over a dedicated communication path, it is called circuit switching.

Ex: Phone calls

- This technique was somewhere in middle of circuit switching and packet switching. In message switching, the whole message is treated as a data unit and is switching / transferred in its entirety.

- Shortcomings of message switching gave birth to an idea of packet switching. The entire message is broken down into smaller chunks called packets. The switching information is added in the header of each packet and transmitted independently.

- Which protocol is commonly associated with the Data Link Layer?

Ans. Ethernet

- What are the two sub-layers of the Data Link Layer?

Ans. Logical link & Media Access

- Which of the following devices operates at the Data Link Layer?

Ans. Switch

- Data link layer does many tasks on behalf of upper layer. These are:

  - Framing

  - Addressing

  - Sychronization

  - Error control

  - Flow control

  - Multi access

- Flow Control:

  - Stop and wait

  - Sliding window

- Error control:

  - Stop and wait ARQ

  - Go-back-N ARQ

  - Selective Repeat ARQ

- RMON stands for Remote Network Monitoring. It is an extension of the Simple Network Management Protocol (SNMP) that allows detailed monitoring of network statistics for Ethernet networks.

- Which layer of the OSI model does RMON operate?

Ans. Network layer

- What is an RMON probe?

Ans. A device for capturing packets

- The principle used in the token ring network is that a token is circulating in the ring, and whichever node grabs that token will have the right to transmit the data.

- What is the maximum number of devices that can be connected in a Token Ring network?

Ans. 255

- What type of signaling is used in a Token Ring network?

Ans. Baseband

- What is the main purpose of Hamming Code?

Ans. Error detection & correction

- How many parity bits are needed for a Hamming Code that covers 'k' data bits?

Ans. log2(k+r)

- In Hamming Code, what does the term 'redundancy' refer to?

Ans. Excess data bits

- What is the maximum number of errors that Hamming Code can correct?

Ans. 1

- Byte Stuffing: A byte is stuffed in the message to differentiate from the delimiter. This is also called character-oriented framing.

- Bit Stuffing: A pattern of bits of arbitrary length is stuffed in the message to differentiate from the delimiter. This is also called bit oriented framing.

- Which character is commonly used as a flag in byte stuffing?

Ans. 0x7E

- Which byte is often used as an escape character in byte stuffing?

Ans. 0x7D

- Media Access Control (MAC) address is a physical address that works at the data link layer of the OSI model.

- A MAC address is a 48 or 64-bit address associated with a network adapter.

- MAC  a. Unicast  b. Multicast  c. Broadcast

- 00-14-22  DELL ,   00-04-DC  Nortel , 3C-5A-B4  Google, Inc.

- Which polynomial is commonly used in CRC calculations?

Ans. X^8 + X^7 + X^4 + X^3 + 1

- What does the term 'frame check sequence' (FCS) refer to in the context of CRC?

Ans. The section of data used to detect errors

- What is the primary purpose of framing in data communication?

Ans. To encapsulate data into packets

- Which framing technique uses flags to delimit frames?

Ans. Bit-oriented framing

- For m data bits, r redundant bits are used. r bits can provide 2r combinations of information. In m+r bit codeword, there is possibility that the r bits themselves may get corrupted. So the number of r bits used must inform about m+r bit locations plus no-error information, i.e. m+r+1.             2^r  >= m + r + 1

- Error-correcting codes (ECC) are a sequence of numbers generated by specific algorithms for detecting and removing errors in data that has been transmitted over noisy channels.

- Error Correcting Codes  1. Block Code  2. Convolutional code

- Hamming Codes It is a block code that is capable of detecting up to two simultaneous bit errors and correcting single-bit errors.

- Binary Convolution Code Here, an encoder processes an input sequence of bits of arbitrary length and generates a sequence of output bits.

- Reed - Solomon Code They are block codes that are capable of correcting burst errors in the received data block.

- Low-Density Parity Check Code It is a block code specified by a parity-check matrix containing a low density of 1s. They are suitable for large block sizes in very noisy channels.

- Layer-3 in the OSI model is called Network layer. Network layer manages options pertaining to host and network addressing, managing sub-networks, and internetworking.

- Network layer takes the responsibility for routing packets from source to destination within or outside a subnet.

- Routing:

  - Unicast  Distance vector routing protocol  Link state Routing protocol

  - Broadcast  Distance Vector Multicast Routing Protocol  Multicast Open Shortest Path First  Core Based Tree  Protocol independent Multicast

  - Multicast

  - Anycast

- Routing between two networks is called internetworking.

- Routing protocols which are used within an organization or administration are called Interior Gateway Protocols or IGP. RIP, OSPF are examples of IGP. Routing between different organizations or administrations may have Exterior Gateway Protocol, and there is only one EGP i.e. Border Gateway Protocol.

- Tunneling is a mechanism by which two or more same networks communicate with each other, by passing intermediate networking complexities. Tunneling is configured at both ends.

- What is the main function of a gateway in networking?

Ans. To connect different protocol network

- Routing Information Protocol'' (RIP) is a distance-vector protocol which uses hop count as its primary metric. It governs the router and directs them to share information with the neighbouring routers connected to the local area network.

- The hop count can be defined as the number of routers between the source and destination. The maximum number of hops allowed in a RIP network is 15 which limits RIP to small networks only (local area network).

- What does BGP stand for?   Border gateway protocol

- What is the main purpose of BGP?

Ans. To route data between different autonomous systems

- Which algorithm does BGP primarily use to make routing decisions?

Ans. Bellman-Ford Algo

- What does OSPF stand for?  Open shortest path first

- What type of routing protocol is OSPF?  Link State

- Which algorithm does OSPF utilize to determine the shortest path?   Dijkstra’s Algo

- What is the maximum area a single OSPF router can belong to?  Unlimited

- What does OSPF use to reduce routing overhead?  Route Summarization

- Which type of NAT allows multiple devices to be mapped to a single public IP address? Port Address Translation (PAT)

- Network Address translation is used to translate Private IP address to Public IP address to communicate LAN side of the Device to Global Network. Network address translation can be processed in Router or Firewall.

- What is the primary function of the transport layer in the OSI model?

Ans. end-to-end communication

- Which protocol is known for providing reliable data transmission?

Ans. TCP

- TCP  reliable, Connection oriented

- UDP  unreliable, connectionless

- UDP Application:

  - Domain Name Services

  - Simple Network Management Protocol

  - Trivial File Transfer Protocol

  - Routing Information Protocol

  - Kerberos

- What is congestion control in computer networks?

Ans.  A technique to manage network traffic

- Which algorithm is commonly used for congestion control in TCP?

Ans. Additive Increase Multiplicative Decrease

- Which of the following is a common cause of network congestion?

Ans.  Sudden increase in traffic

- Transport layer security protocol is one of the security protocols which are designed to facilitate privacy and data security for communications over the Internet. The main use of TLS is to encrypt the communication between web applications and servers, like web browsers loading a website.

- What is the first step in the TLS handshake process?

Ans. Client Hello

- During the TLS handshake, what does the server send back to the client after 'Client Hello'?

Ans. Server Hello

- What is the purpose of the 'Client Key Exchange' message?

Ans. To establish the session keys

- What happens in the final step of the TLS handshake?

Ans. Client sends Finished message

- TCP examples: HTTP, HTTPs, FTP, SMTP, and Telnet use TCP.

- UDP examples: DNS, DHCP, TFTP, SNMP, RIP, and VoIP use UDP.

- Which protocol guarantees data delivery? TCP

- The Domain Name System (DNS) works on Client Server model. It uses UDP protocol for transport layer communication. DNS uses UDP port 53.

- The Simple Mail Transfer Protocol (SMTP) is used to transfer electronic mail from one user to another. SMTP uses TCP port number 25 and 587.

- The File Transfer Protocol (FTP) is the most widely used protocol for file transfer over the network.

- FTP uses out-of-band controlling i.e. FTP uses TCP port 20 for exchanging controlling information and the actual data is sent over TCP port 21.

- The Post Office Protocol version 3 (POP 3) is a simple mail retrieval protocol used by User Agents (client email software) to retrieve mails from mail server.

- The Hyper Text Transfer Protocol (HTTP) is the foundation of World Wide Web. Hypertext is well organized documentation system which uses hyperlinks to link the pages in the text documents. HTTP works on client server model.

- VPN stands for Virtual Private Network. It allows you to connect your computer to a private network, creating an encrypted connection that masks your IP address to securely share data and surf the web, protecting your identity online.

- VPN  a. Router VPN  b. Firewall VPN  c. Network os

- It is one of the approaches to congestion control. Router contains a buffer to store packets and route it to destination. When the buffer is full, it simply discards some packets. It chooses the packet to be discarded based on the strategy implemented in the data link layer. This is called load shedding

- Which HTTP method is used to request data from a server?	GET

- Which status code indicates that a request was successful?         200

- HTTP is a connectionless protocol. It is a stateless protocol.

- HTTP is considered connectionless at the application layer because it follows a request-response model where the connection is dropped after each request. However, it is connection-oriented at the transport layer, as it uses TCP to establish a reliable, dedicated path between client and server.

- File Transfer Protocol (FTP) is an application layer protocol that is used to transfer the files between the local devices (PC, smartphone, etc.) to a server. It transfers both text and binary files over the Internet.

- FTP opens two connections between the computers one for the commands and replies (control connection) and a second one for data transfers (data connection).

- A Control Connectcion is established on Port number 21. Data Connection is initiated on Port number 20.

- Which command is used to upload files in FTP?   PUT

- Secure Sockets Layer (SSL) is a standard technique for transmitting documents securely across a network. SSL technology, created by Netscape, establishes a secure connection between a Web server and a browser, ensuring private and secure data transmission. SSL communicates using the Transport Control Protocol (TCP).

- The term "socket" in SSL refers to the method of sending data via a network between a client and a server.

- Data integrity Information is safe from tampering. The SSL Record Protocol, SSL Handshake Protocol, SSL Change CipherSpec Protocol, and SSL Alert Protocol maintain data privacy.

- What type of encryption does SSL use?  Symmetric & Asymmetric

- What is the maximum throughput of the Slotted ALOHA protocol?  36.8%

- Aloha is a packet switching system. The time interval required to transmit one packet is called a slot. Aloha is a random access technique.

- The mode of random access in which users can transmit at any time is called pure Aloha. It means, in Pure ALOHA the channel utilization is 18 percent.

- Psuccess = exp(-TG) =  S/G     S= Ge(power)-TG

- A sliding window is a method for controlling sending data packets between two network devices where dependable and sequential delivery of data packets is needed, such as using the Data Link Layer (OSI model) or Transmission Control Protocol (TCP).

- Go-Back-N Automatic Repeat Query (ARQ) protocol is also referred to as Go-Back-N Automatic Repeat Request. It is a data link layer protocol that helps a sliding window method. In this, if any frame is manipulated or lost, all subsequent frames have to be sent again. All the frames are numbers to deal with the most and duplicate frames. If the sender does not receive the receivers acknowledgement, then all the frames available in the current window will be retransmitted.

- In this protocol, the sender window size is always similar to the size of the receiver window. The size of the sliding window is continually greater than 1. If the receiver obtains a corrupt frame, it does not directly remove it. It sends a negative acknowledgement to the sender. The sender sends that frame again immediately, receiving a negative acknowledgement. There is no waiting for any time-out to share that frame.  Selective Repeat ARQ

- Stop & Wait Protocol:

- What is the primary purpose of link state routing protocols? To find shortest path

- Which of the following protocols is an example of a link state routing protocol? OSRF.

- What type of graphs can Dijkstra's algorithm be applied to? Directed & Undirected

- Which data structure is often used to implement Dijkstra's algorithm? Priority Queue

- Adaptive routing algorithms, also known as dynamic routing algorithms

- Non-adaptive Routing algorithms, also known as static routing algorithms

- When too many packets are present in the network it causes packet delay and loss of packet which degrades the performance of the system. This situation is called congestion.

- The network layer and transport layer share the responsibility for handling congestions.

- What does 'SSID' stand for in wireless networking? Service set Identifier

- What frequency bands are commonly used in wireless networks? 2.4GHZ – 5GHZ

- What does WLAN stand for? Wireless LAN

- Which of the following is a common standard for WLAN? IEEE 802.11

- What frequency bands are generally used by WLANs? 2.4 – 5GHZ

- What is the maximum data rate of 802.11ac? 1.3Gbps

- What technology is used to reduce interference in WLANs? MIMO (Multiple Input Multiple Output)

- Wireless LANs are those Local Area Networks that use high frequency radio waves instead of cables for connecting the devices in LAN. Users connected by WLANs can move around within the area of network coverage. Most WLANs are based upon the standard IEEE 802.11 or WiFi.

- Wireless LAN Is slower than wired LAN

- What does IEEE stand for? Institute of Electrical & Electronics Engineers

- Multiplexing is the process of combining multiple signals into one signal, over a shared medium. If analog signals are multiplexed, it is Analog Multiplexing and if digital signals are multiplexed, that process is Digital Multiplexing.

- Which type of signals typically use Synchronous TDM? Digital

- In the Synchronous Time Division Multiplexing (STDM), the multiplexer assigns an equal time slot to every device at all times, whether or not a device has anything to send.

- The switch transfers from device to device at a fixed price and in a permanent order. This process is called interleaving.

- What type of applications benefit most from Asynchronous TDM? interactive application

- Which of the following is an example of FDM? Radio Broadcasting

- TDM and FDM are both multiplexing techniques. FDM is meant for analog signals, while TDM works well with both analog as well as digital signals. However, TDM is mostly used in digital communications only. In TDM, syncronization pulse is important; whereas in FDM, Guard Band is required.

- What is the typical wavelength range used in WDM systems? 1480nm-1625nm

- Course WDM (CWDM) : CWDM generally operates with 8 channels where the spacing between the channels is 20 nm (nanometers) apart. It consumes less energy than DWDM and is less expensive. However, the capacity of the links, as well as the distance supported, is lesser.

- Dense WDM (DWDM) : In DWDM, the number of multiplexed channels much larger than CWDM. It is either 40 at 100GHz spacing or 80 with 50GHz spacing. Due to this, they can transmit the huge quantity of data through a single fiber link. DWDM is generally applied in core networks of telecommunications and cable networks. It is also used in cloud data centers for their IaaS services.

- What does ISDN stand for? Integrated Service Digital Network

- What is the key advantage of using Selective Repeat ARQ compared to Go-Back-N?

Higher Throughput

- Which of the following is a disadvantage of Selective Repeat ARQ?

Complexity in buffer management

- Which of the following is a characteristic of flooding? Potential for network congestion

- In which scenario is flooding most beneficial? When the network topology is unknown

- What is a disadvantage of flooding? It may cause loops

- In which format are email messages typically encoded? ASCII UTF-8 BASE64

- In asymmetric or public-key cryptography, there are two keys: a private key and a public key. The private key is kept by the receiver. The public key is announced to the public.

- In symmetric-key cryptography, the same key is used by both parties. The sender uses this key and an encryption algorithm to encrypt data; the receiver uses the same key and the corresponding decryption algorithm to decrypt the data.

- Casting in computer networks means transmitting data (stream of packets) over a network.

- In Unicast transmission, the data is transferred from a single sender (or a single source host) to a single receiver (or a single destination host).

- In Broadcast transmission, the data is transmitted from one or more senders to all the receivers within the same network or in other networks. This type of transmission is useful in network management packets such as ARP (Address Resolution Protocol) and RIP (Routing Information Protocol) where all the devices must see the data.

- When the data is transmitted from a single source host to a specific group of hosts having the interest to receive the data, it is known as multicast transmission.

- Which protocol is commonly used for multicast communication? TCP

- Network virtualization represents the administration and monitoring of an entire computer network as a single administrative entity from a single software-based administrators console.

- Congestion occurs in a network when it receives too many packets than its designed capacity. Network congestion occurs in case of traffic overloading.

- Flow control is a system that guarantees a sender's transmission rate is proportional to the receiver's reception capability.

- Data is transmitted as a continuous stream in synchronous transmission, while in the asynchronous transmission method, data is transferred as discrete chunks of packets.

- What type of applications typically use asynchronous transmission?  Keyboard and mouse inputs

- A Denial of Service (DoS) attack is an attempt to make a network resource or service unavailable to legitimate users by overwhelming it with traffic from multiple sources. The goal of a DoS attack is to disrupt the normal operation of a network or website, making it unavailable to legitimate users. DoS attacks can be launched from a single machine or from a distributed network of compromised machines, known as a botnet.

- A Distributed Denial of Service (DDoS) attack is a type of attack that is similar to a Denial of Service (DoS) attack, but it uses a distributed network of compromised machines to generate traffic to overwhelm a target. This makes DDoS attacks much more difficult to defend against than traditional DoS attacks because the traffic is coming from multiple sources, making it difficult to distinguish legitimate traffic from attack traffic.

- Phishing is a type of social engineering attack that attempts to trick individuals into revealing sensitive information, such as login credentials or financial information. The goal of a phishing attack is to steal sensitive information or gain unauthorized access to a user's accounts or devices.

- A Man-in-the-Middle (MitM) attack is a type of attack in which an attacker intercepts and modifies network traffic between two parties without their knowledge.

- A sniffing attack is a type of network attack that involves intercepting and analyzing network traffic in order to extract sensitive information.

- SQL injection is a type of attack that exploits vulnerabilities in the SQL code of websites or web-based applications.

- A Remote Code Execution (RCE) attack is a type of attack that allows an attacker to execute arbitrary code on a remote system. This can allow the attacker to take control of the system, steal sensitive information, or cause damage to the system and its resources.

- What does WiMAX stand for?  Wireless Interoperability for Microwave Access

- Buffering refers to the process of temporarily storing data in memory (a buffer) before it is sent or received over a network. The buffer helps to smooth out any variations in the rate at which data can be sent or received, and ensures that the sender and receiver can work at their own pace without interruption.

- Authentication in computer networks is the process of verifying the identity of a device or user on a network.

- Passwords As mentioned earlier, passwords are a common method of authentication. Users are prompted to enter a combination of their username and password in order to log in to a system or access network resources.

- Biometric authentication This method relies on unique physical characteristics of the user, such as fingerprints, facial recognition, or iris scans, to verify their identity. Biometric authentication can be more secure than passwords, as it is difficult to forge or steal someone's fingerprints or facial features.

- Security tokens Security tokens are physical devices that generate one-time codes or passwords that can be used to log in to a system. The codes generated by the security token are typically only valid for a short period of time and can't be used again, adding an additional layer of security.

- Two-factor authentication (2FA) This method requires users to provide two different types of credentials in order to log in. For example, a user might be prompted to enter their password and then confirm their identity by entering a code sent to their phone.

- Certificates and PKI Some networks use certificates or public key infrastructure (PKI) to verify the identity of devices. In these systems, each device is issued a unique digital certificate that is used to authenticate its identity.

- Single sign-on (SSO) SSO systems allow users to log in to multiple applications with a single set of credentials. This can make it easier for users to access the resources they need, while still maintaining strong security controls.

- Smart cards Smart cards are physical cards that contain a chip that stores information about the user, such as their credentials or other identifying information. Users can use smart cards to log in to systems or access network resources by inserting the card into a card reader.

- One-time passwords (OTP) OTPs are passwords that are valid for only a single use. They are often used in conjunction with other authentication methods, such as passwords or security tokens. OTPs can add an additional layer of security, as they can't be used again once they have been used to log in.

- Multi-factor authentication (MFA) MFA requires users to provide multiple types of credentials in order to log in. This can include passwords, security tokens, biometric authentication, and more. MFA can be more secure than other methods, as it requires multiple types of credentials to be provided.

- Tokenless multi-factor authentication This method allows users to authenticate their identity using multiple factors without the need for a physical token. For example, a user might be prompted to enter their password and then confirm their identity by answering security questions or using biometric authentication. Tokenless MFA can be more convenient for users, as they don't need to carry a physical token with them in order to log in.

- SMS-based authentication In this method, users are sent a code via text message that they must enter in order to log in to a system. SMS-based authentication can be a convenient way to provide an additional layer of security, as it requires the user to have access to their phone in order to log in.